OpenRacer

Works offline. Amplifies online. Open source.

Project specification · v0.6 · April 2026

## What changed in v0.6

Three additions reflecting the reality of modern sailing: Cardigan Bay has no cellular signal, but Grand Prix racing yachts now carry Starlink. Navigation matters when not racing. And we need a deep spec for the future cruising platform.

- Connectivity as a spectrum, not a toggle. Three modes — offline, patchy, constant — with graceful degradation down and graceful amplification up. Starlink-equipped yachts get real-time fleet tracking, live AI coaching, live AIS via MCP. Phone-only dinghies at sea get the full racing experience with zero signal.

- Navigation mode for v1. Lightweight cruising interface: waypoint nav, MOB, anchor alarm, route planning, depth alarm, night mode, basic AIS, digital logbook. Sibling to race mode with clean toggle.

- Deep future spec. Part 11 lays out how OpenRacer extends into a full cruising platform competing with OpenCPN and Navionics. Not v1 scope, but designed for from day one.

## Executive summary

OpenRacer is an open-source racing and navigation system that works with any existing marine electronics, starting from just a smartphone. Three product tiers (free app, Raspberry Pi, full hardware kit) share one codebase.

Two primary use cases: racing and general navigation. Race mode delivers fast course entry, live tactical nudges, post-race AI coaching. Cruise mode delivers waypoint nav, MOB, anchor alarms, route planning.

Connectivity-adaptive: racing works fully offline on phone GPS. Patchy signal queues cloud features. Constant connection (Starlink, coastal 5G, in-harbour) unlocks real-time fleet tracking, live AI coaching during the race, live AIS overlays, streaming telemetry.

v1 scope is 60 weeks to community launch. Part 11 sketches the post-v1 cruising platform that extends us into Navionics/OpenCPN territory.

# Part 1 · Course entry — the hero feature

Detailed in v0.4 and v0.5, summarised here.

## Seven input methods

| Method | Best when | Taps | Time |
| --- | --- | --- | --- |
| Committee-boat push | Committee runs OpenRacer | 1 | 3 sec |
| Pick from library | Fixed club marks | 1 | 2 sec |
| Chart seamarks | Cardinal in OpenSeaMap | 1 | 3 sec |
| Drop at current GPS | Sailing past mark | 1 | 2 sec |
| Point phone at mark | Can see, not near | 3 | 8-40 sec |
| Bearing + distance | “280° 0.4nm from CB” | 5 | 15 sec |
| Tap on chart | Know roughly where | 2 | 4 sec |

## Mark lifespan tiers

Four tiers ordered by likelihood of use:

- Club seasonal — laid April, removed October

- Chart permanent — OpenSeaMap nav marks, cardinals

- Race-day recent — last 14 days

- Single-race temporary — auto-deleted after race

## Committee-boat push

mDNS over WiFi + Bluetooth LE. SAP Buoy Pinger schema for interop. Signed bundles prevent spoofing. One-tap accept.

## Course templates

Windward-Leeward, Olympic, Triangle, Trapezoid, Round-the-cans, Custom. Downstream AI uses templates to understand leg intent.

# Part 2 · Connectivity — offline foundation, online amplification

One architecture, three scenarios Racing at Abersoch with no signal: full function, no degradation. Cruising in a marina with patchy 4G: cloud features queue. Racing a Grand Prix yacht with Starlink: real-time AI coaching, live fleet AIS, streaming telemetry to shore. Same codebase handles all three.

## The three connectivity scenarios

| Mode | Typical scenario | OpenRacer behaviour |
| --- | --- | --- |
| Offline | Cardigan Bay, mid-Channel, rural UK coast | Full racing. Cached weather/tides. Local AI Q&A. Logs queue for later sync. |
| Patchy | Most club venues — intermittent 4G | Racing offline-first. Queued cloud requests execute in bursts when signal returns. |
| Constant | Starlink yacht, in-harbour, strong 5G coastal, satellite iPhone | Real-time AI coach during race. Live AIS. Live fleet tracking. Streaming telemetry. Cloud debriefs in 15 seconds. |

## The design principle

GPS is satellite-based. Phone sensors are local. Marine instruments are on the boat. Almost everything that matters for a race is achievable with zero connectivity — and works better with connectivity.

Every feature has three defined behaviours (offline / patchy / constant) rather than two (works / broken). This reframes development: we don't “add offline support” as an afterthought, we build features that automatically choose the right mode.

## Features categorised by connectivity tier

### Works fully offline — the core

- GPS, course, speed, track, boat sensors

- All marks, course building, committee-boat push (peer-to-peer)

- Race timer, start line, laylines, VMG, polar performance

- Tidal predictions (XTide on-device, worldwide)

- Cached weather (most recent online snapshot)

- Real-time nudges including gust-aware tactics

- Local AI debrief (on-device Llama 3.2 3B)

- Local AI Q&A (same on-device model)

- Boat learning, personal polars, all trend analysis

- Navigation mode: MOB, anchor, waypoint, route, depth alarm

- Offline charts for pre-downloaded areas

### Enhanced by patchy signal

- Cloud AI debrief (Claude API) replaces local version when available

- Live weather updates (Open-Meteo) replace cached

- Live UKHO/NOAA tidal data augments XTide

- Seasonal mark updates fetch on app launch

- Cross-device sync of user data

- Cloud-based Q&A answers more sophisticated questions

### Amplified by constant connection (Starlink era)

The Grand Prix mode When a boat has constant high-bandwidth connectivity — Starlink Maritime, strong coastal 5G, new satellite-iPhone modes — OpenRacer becomes a qualitatively different product.

- Live AI coaching during the race. Not just post-race debriefs. Ask the AI mid-leg: “we're losing to the boat ahead, what's wrong?” Answer streams in 2 seconds.

- Live AIS via MCP. AI tool queries external AIS services (MarineTraffic, VesselFinder) and surfaces fleet position overlays on your chart in real time.

- Fleet live tracking. Other OpenRacer users in the same race appear as boats on your chart with their positions streaming in. Opt-in per race.

- Streaming telemetry to shore. A coach or team manager ashore watches your full boat data stream. Post-tack analysis happens in parallel with you sailing.

- Live weather model updates. ECMWF refreshes every 6 hours. With constant signal, the forecast on your chart is always current. Mid-race front shifts become visible.

- Video overlay streaming. Phone camera feed + data overlay streamed to coaches or spectators. Post-race video debrief generates automatically.

- Cross-boat tactical sharing. Fleet boats share tactical data: who tacked when, who saw what wind, where the lifts are. Anonymised and opt-in.

## Architectural rules

- Never block on network. Any network fetch has either a local fallback or returns immediately with a pending-update marker.

- Queue cloud requests. Persisted queue, executes when signal available, ordered by priority.

- Stale data shows freshness. Subtle badge, never alarming. Racing data never shows stale; that's always live from sensors.

- Local models available. On-device AI for Q&A and debriefs ensures every feature has an offline path.

- Local first, cloud is sync target. Never wait for the cloud to know what the user just did.

- Opportunistic sync. Background sync when signal returns, user never triggers manually.

## Testing approach

CI runs all feature tests with network disabled. Phase 1 exit gate: 2-hour full race completed in airplane mode at Abersoch SC, no degraded UX visible to the sailor. Later, a separate Starlink-mode test matrix validates the constant-connection amplification features.

# Part 3 · Navigation mode

When you're not racing Between races, on the delivery sail, during cruising week, crossing open water. OpenRacer needs to be more than a race tool. v1 ships a lightweight but genuinely useful cruising interface.

## The mode model

Top-level toggle: Race / Cruise. Same codebase, same data, different defaults. Each surfaces the features that matter for its activity.

### Race mode defaults

- VMG, AWA/AWS, TWA/TWS, laylines, start timer

- Alerts: gust-header, gust-lift, start-line anomaly, performance drift

- Chart: race marks, laylines, tidal arrows on course

- Auto-debrief at race finish

### Cruise mode defaults

- COG, SOG, heading, depth, next waypoint bearing/distance/ETA

- Alerts: depth below threshold, anchor drift, AIS collision risk, waypoint arrival

- Chart: waypoints, route, AIS targets, tidal arrows on area

- Auto trip log

### Switching

- Manual: tap mode selector top-right

- Auto-arm: race timer activation switches to Race mode

- Auto-release: 15 min after finish, offer Cruise mode

- Long-press clock: quick toggle

## Cruise mode features in v1

### Waypoint navigation

Tap chart, pick mark, or enter coordinates. Display shows bearing, distance, ETA by current SOG, cross-track error if on route. Arrival alert inside user-set radius (default 50m).

### Route planning

Multi-waypoint routes, tap marks in sequence or draw on chart. Save for reuse. v1: minimum-distance only. Weather routing deferred to Part 11.

### Man overboard

Large red MOB button persistent in cruise mode. One tap: records position, starts elapsed timer, shows live bearing/distance back, large return-compass, 30-second audio countdowns, two-tap cancel required.

### Anchor alarm

Set current position as anchor, configure radius (default 50m). GPS drift outside radius triggers audio alarm + push notification. Smart default: 5 min stationary + app backgrounded → offer auto-arm.

### Depth alarm

User-set threshold (default 3m under keel). Audio alert when depth drops below. Requires depth sensor (Tier 2/3).

### Trip log

Auto-captured in cruise mode. Distance, time, avg speed, max speed, route, conditions. Summary on arrival, exportable GPX/PDF.

### Night mode

Red-tinted UI, auto-enables at local sunset, manual override available.

### Basic AIS display

If AIS data is available (NMEA input or Starlink-era live feed via MCP), targets appear with MMSI, name, heading, speed. CPA/TCPA deferred to Part 11. Display only in v1.

### Digital logbook

Optional hourly auto-entries: position, time, weather, conditions, notes. PDF export for Coast Guard / insurance. Standard marine format.

## What cruise mode does NOT do in v1

Deferred to Part 11:

- Weather routing (multi-waypoint optimisation)

- GRIB file download and overlay

- Full S-57 vector / S-63 encrypted charts

- Radar overlay

- NMEA 2000 engine, tanks, environment monitoring

- Autopilot control (PyPilot already handles)

- DSC / VHF integration

- Satellite comms beyond iPhone Starlink

- Crew watch scheduling

- Float plan submission

v1 cruise mode is enough for delivery sails, weekend cruising, and venue transits. Part 11 describes the ambition beyond that.

# Part 4 · Data ecosystem

## Five streams

| Stream | Source | What it enables |
| --- | --- | --- |
| Boat state | Phone / instruments / our sensors | Position, speed, heading, wind |
| Chart seamarks | OpenSeaMap, OSM | Pre-populated marks, chart display |
| Tidal | UKHO / NOAA / XTide offline | Current, tide height, stream arrows |
| Weather | Open-Meteo + cache | Wind, waves, gusts, shifts |
| Historical | Saved sessions + opt-in fleet | Polars, tacking losses, heuristics |

## Offline chart management

Charts are the largest offline-critical data category. Users download areas before setting off; the system prompts intelligently when signal is good.

### Chart packages

- Pre-defined regions — “North Wales”, “Solent”, etc. Size estimates shown. Typically 50-300 MB.

- Custom rectangle — draw on chart, estimate shown.

- Route-based — corridor along planned route plus buffer.

### Smart prompts

- “Near edge of downloaded area — extend before leaving?”

- “Route crosses undownloaded areas — download now?”

- “Still online at marina — top up today's area?”

### Auto-caching

~5nm radius of current GPS cached in background when signal is good. Covers “I forgot to download but I'm still in the marina.”

### Storage management

Management screen: total usage, list with last used / updated, recommended removals, update indicators, one-tap clear-all-except-current.

## Tidal data

XTide embedded for worldwide offline coverage. UKHO Admiralty (UK, 10k free req/month). NOAA (US, free). Tidal stream arrows overlay chart at diamond positions.

## Weather data

Open-Meteo primary. Free, AGPLv3, ECMWF wave model at 5km European resolution, hourly updates, 16-day forecast. Cache all 16 days locally, serve from cache with freshness badge, refresh in background.

# Part 5 · AI layer

Four uses of AI, each with offline, patchy, and constant-connection paths.

## Q&A

- Constant: Claude API streaming, mid-race live

- Patchy: Claude API with 2-3 sec response, ~0.5p/question

- Offline: Llama 3.2 3B on-device, 5-15 sec, six question categories

## Gust-aware tactical nudges

Deterministic signal processing on sensor data. Fully offline. Two triggers: gust-with-header (tack) and gust-with-lift (press on). Detection: 20%+ AWS spike over rolling 10-sec avg, track AWA shift over next 20 sec. Audio + haptic + visual cues. Layline recalibration when shifts persist 3+ min.

## Real-time nudges (other)

All offline: performance drift, start-line anomaly, tide change, safety. Max 4 nudges per leg on average.

## Post-race debrief

- Offline at finish: Local model debrief in 2-3 min

- Patchy at finish: Cloud debrief in 30 sec

- Constant at finish: Cloud debrief in 15 sec, with live-race context already analysed

When signal returns after an offline finish, cloud version auto-generates and upgrades the local one.

## Cost

~20p per cloud debrief, ~1p per Q&A. ~£7/year for a 30-race sailor. Bring-your-own-API-key default; optional £15/year subscription.

# Part 6 · MCP layer

OpenRacer ships an MCP server. Power users point their own Claude Pro or ChatGPT at it for AI-driven analysis.

## What MCP exposes

- Resources: boat state, polar, sessions, tracks, debriefs, marks, tides, weather

- Tools: set_mark, start_race_timer, switch_display_layout, analyze_session, export_session

- Prompts: compare_sessions, trend_last_month, identify_weakness, pre_race_briefing

## Starlink-era: Live AIS integration

With constant connection, MCP becomes a gateway to live external services. Example: the AI tool queries MarineTraffic's MCP server for AIS data, ingests that data into its reasoning about the user's race, and surfaces live fleet positions on the chart — without OpenRacer itself having AIS hardware. This is the kind of cross-service AI reasoning that's only possible with MCP.

## Security

- Local (default) — same-device AI tools, no auth

- LAN — authorised via QR code

- Remote — OAuth tokens, revocable, all access logged

# Part 7 · Open API

Distinct from MCP. Traditional REST + WebSocket + webhooks + OAuth, builds on SignalK's existing API.

## Use cases

- Club website: live race standings, leaderboard

- Sail-maker: tuning guides with boat data

- Coaching business: aggregate multiple sailors

- Cross-platform: other apps consume OpenRacer data

- Hardware: new sensors without SignalK config

## Starlink-era: streaming telemetry

With constant connection, the Open API unlocks shore-based real-time monitoring. A coach's browser opens a URL and watches the boat's full data stream in real time. Commercial racing programmes use this to have shore-side tacticians analysing conditions and texting the on-water team. We enable it for everyone via open standards.

# Part 8 · Boat learning

Over a season, OpenRacer learns your boat, sailing, and venues. All computation is local; cloud sync is opt-in for cross-device.

## What gets learned

- Polar calibration — your actual curve, typically 5-15% different from published

- Tacking / gybing losses — statistically meaningful after 100+ tacks

- Start-line behaviour — typical timing and position patterns

- Venue heuristics — which side pays in which wind at each venue

- Gust-handling signature — how you respond to gusts

- Fleet comparison (opt-in, online)

## Feedback loop

Learning feeds back to live. Your personal polar drives laylines. Your tacking loss distribution informs start-line buffers. Venue heuristics surface as gentle suggestions.

# Part 9 · Architecture

## The stack

| Layer | Component | Tech | Role |
| --- | --- | --- | --- |
| L0 | Sensors | NMEA / BLE | Instruments |
| L1 | Mobile app | React Native + TS | Tier 1 + control surface |
| L2 | Pi distribution | Debian + SignalK | On-boat server |
| L3 | SignalK server | Node.js | Data hub |
| L4 | Core plugin | TypeScript | Display orchestrator, race calcs, nav calcs |
| L5 | E-ink renderer | SSR React | Kindle pages |
| L6 | Data enrichment | Node/Python | Tidal, weather, chart sync |
| L7 | Analytics & learning | Python + DuckDB | Sessions, polars, trends |
| L8 | AI coaching | Python + Claude + local Llama | Q&A, debriefs, gusts |
| L9 | MCP server | Python or TS | AI tool integration |
| L10 | P2P sharing | mDNS + BLE | Committee-boat push |
| L11 | Open API | Node.js on SignalK | Third-party integrations |
| L12 | Offline cache manager | Platform-native | Chart tiles, queues, sync |

L12 is new in v0.6 — the cross-cutting offline cache layer underneath everything. Manages tile storage, queued cloud requests, cached API responses, background sync.

## Data storage

- DuckDB — analytics queries

- SQLite — transactional data

- MBTiles — offline charts

- Platform cache APIs — opportunistic network caching

## Privacy

- Local by default

- Cloud AI opt-in per feature

- Fleet sharing opt-in per race

- Full export, deletable any time

# Part 10 · Development plan

| Phase | Duration | Deliverable | Gate |
| --- | --- | --- | --- |
| 0 | 2w | Skeleton + offline foundations | iOS/Android, airplane-mode GPS works |
| 1 | 8w | Course entry hero + race timer + start line | Full race in 3 min course-to-start, airplane mode |
| 2 | 4w | Chart upload, polars, VMG, track logging, offline chart packages | Race start-to-finish all offline |
| 3 | 4w | Navigation mode v1 + tidal + weather overlays | MOB, anchor, waypoint, route work offline |
| 4 | 6w | AI debriefs (cloud + local) + gust nudges | 30s online / 3min offline debrief; gusts in live race |
| 5 | 4w | NL Q&A (cloud + local) | Ask-the-boat online and offline |
| 6 | 6w | Pi distribution + auto-discovery wizard | Pi detects NMEA unaided |
| 7 | 8w | Tier 2 display orchestrator + nudge tuning | Multi-Kindle in race, nudges validated |
| 8 | 4w | MCP + Open API + webhooks | Claude Desktop queries; third-party webhook fires |
| 9 | 6w | Boat learning + venue heuristics | Personal polar differs after 10 sessions |
| 10 | 4w | Tier 3 hardware kit + IP68 hardening | First Tier 3 boat racing |
| 11 | 4w | Docs, packaging, community launch | GitHub release, first contributor |

Total: 60 weeks (2 weeks more than v0.5 for nav mode v1). First shippable alpha at week 10 — already better than competitors at course entry.

## Phase 3 in detail (nav mode v1)

- Week 13-14: Cruise/Race toggle, cruise defaults

- Week 14-15: Waypoint nav, route planning, cross-track calcs

- Week 15-16: MOB, anchor alarm, depth alarm framework

- Week 16: Night mode, trip log, logbook, basic AIS display

- Weeks 13-16 parallel: tidal overlays, Open-Meteo, offline chart packages

# Part 11 · Future — OpenRacer as a full navigation platform

The five-year vision v1 is a great racing app with basic cruising. v2-v4 extend into a complete open-source cruising platform competing with OpenCPN and Navionics. Not in scope for the first 60 weeks, but designed for from day one so we don't paint ourselves into a corner.

## Market map

| Category | Examples | Position |
| --- | --- | --- |
| Proprietary MFDs | Raymarine Axiom, B&G Zeus, Garmin | Expensive, brand-locked, closed |
| Subscription plotters | Navionics, iNavX, Aqua Map | £50-200/yr, mobile-focused |
| Open-source desktop | OpenCPN, QGIS | Free, powerful, desktop-oriented |
| Mobile open-source | Almost none mature | The gap we fill |

OpenRacer's racing reputation (if v1 succeeds) creates the user base to grow into cruising — sailors who used it Wednesday nights keep it during cruising week.

## Feature areas post-v1

### Charts — the hardest problem

- OpenSeaMap + OSM — free community, our foundation

- KAP / BSB raster — gov-produced raster (NOAA US free, UKHO UK fee)

- MBTiles — generic tile container

- S-57 vector — IHO standard, requires dedicated renderer

- S-63 encrypted — commercial charts, UKHO-issued permits

- oeSENC — o-charts encrypted, OpenCPN uses it

### AIS with full collision avoidance

- CPA/TCPA Closest Point of Approach / Time to CPA — alarm when another vessel will come close within user-set time

- Target classification Class A vs B, cargo vs fishing, different alarm profiles

- AIS-SART Search and Rescue Transmitters from MOB devices and life rafts

- DSC integration Digital Selective Calling distress alerts via VHF radio

### Weather routing

Given a route, boat polar, and weather forecast, calculate fastest/safest/most-efficient path. Computationally intensive. Leverage existing open-source (qtvlm, OpenCPN's WeatherRouting plugin) or build our own.

Inputs: GRIB files, our existing polars, tidal stream, user constraints (max wind, daylight only, etc.).

### GRIB weather files

Standard format for numerical weather prediction. NOAA GFS (global, free), ECMWF (commercial), national services. 100 KB to 100+ MB. Download before departure, overlay on chart (wind arrows, pressure contours), feed to weather routing.

### NMEA 2000 full integration

- Engine: RPM, temperature, oil pressure, fuel consumption

- Tanks: fuel, water, waste, oil levels

- Battery: voltage, current, state of charge

- Environment: cabin temp, humidity, barometric pressure

- Navigation lights status and bulb failure

- Autopilot: commands, status, rudder angle

Full N2K turns OpenRacer from racing nav into whole-boat monitoring.

### Autopilot bridge

PyPilot is the open-source autopilot, already integrates with SignalK. Our job: bridge UI. See what pilot is doing, adjust heading/mode, integrate autopilot with route following (“follow route” activates pilot on waypoint bearings).

### Voyage features

- Float plan — formal passage plan submitted to Coast Guard / relatives, auto-updates via satellite

- Formal logbook — legally compliant marine format, auto-populated, PDF export

- Crew management — roster, watch schedule, sleep tracking, safety briefings

- Checklists — pre-departure, MOB drill, fire drill, abandon ship, reef, night handover

### Satellite comms integration

Iridium GO, Starlink Maritime, Garmin InReach, PredictWind Offshore. Position beacons, weather-file download over sat, email gateway, SMS fallback.

## Licensing complexity

- UKHO ADMIRALTY — authoritative UK, commercial redistribution requires licence. AVCS paid-per-vessel. Not feasible at v1 scale.

- Navionics — proprietary, Garmin-owned, can't redistribute. KAP workaround is a legal grey area.

- NOAA — US federal, entirely free, best possible US source.

- OpenSeaMap — open data, CC-BY-SA, less detail than commercial but improving.

- S-63 — requires IHO DPS membership, user permit system, key management overhead.

## Post-v1 development phases

| Phase | Duration | Deliverable | Output |
| --- | --- | --- | --- |
| 12 | 12w | Cruising extension: advanced waypoints, full routes, passage logs | v2 beta to cruising users |
| 13 | 8w | GRIB + weather overlay + basic weather routing | Offshore users adopt |
| 14 | 10w | Full AIS with CPA/TCPA, AIS-SART, DSC | Safety parity with competitors |
| 15 | 8w | NMEA 2000 full bus: engine, tanks, battery | Liveaboards adopt |
| 16 | 6w | Autopilot bridge via PyPilot | Complete autonomous nav |
| 17 | 12w | S-57 vector + oeSENC support | Compete with OpenCPN |
| 18 | 8w | Voyage features: logbook, float plan, crew | Long-distance cruising users |
| 19 | 6w | Satellite comms integration | Offshore adventurers |
| 20 | 4w | Commercial marine features, fleet management | Paid tier for business users |

Post-v1 total: ~74 weeks to full cruising platform. Not committed now; sketched to show architecture can support it.

## Strategic questions before Phase 12

- Is the racing user base (post-v1) big enough to fund cruising development?

- Free or paid tier for cruising?

- UKHO commercial licensing worth the cost?

- Do we have the team to run a project of this scope?

# Part 12 · Decisions outstanding

### 1. React Native stack

Expo SDK 52+, Tamagui UI, Zustand, expo-sqlite, react-native-ble-plx, MapLibre, react-native-background-geolocation, react-native-bonjour.

### 2. Cloud AI provider

Anthropic Claude API primary, abstraction layer for OpenAI or local swap.

### 3. Tidal sources

UKHO Admiralty (UK), NOAA (US), XTide (worldwide offline).

### 4. Hosting

Cloudflare Workers + R2, one VPS for AI pipeline. £20-60/month until 500+ users.

### 5. Name and branding

Confirm OpenRacer availability. Fallbacks: RaceSail, TrimTab, WindwardOS.

### 6. Committee-boat adopter

Who at Abersoch SC volunteers for the first weekend?

### 7. Beta test boat

Yours if possible, otherwise tolerant-crew boat with NMEA 0183.

### 8. Team structure

Solo for Phases 0-5. 2-3 collaborators for 6-9.

### 9. Club admin

Who at Abersoch SC owns the seasonal mark list? Brief them early.

End of specification v0.6